The Digital Pump Section                    5 Dec 2018

Revised 5 Nov 2019

The section of interface will be actively used on 95% of the MPD in the field.

OpCode:  0x03 - DigitalPump to Uart2

(Applies to Wrapped protocol)

Sub OpCode

| SubOpCode (Pump Comms) OpCode=0x03 | Description -Read Timeout | Function |
| --- | --- | --- |
| 0x00 | Zero Wait | No response expected |
| 0x01 | 10msec | 10msec wait for a response |
| 0x02 | 20msec | 20msec wait for a response |
| 0x03 | 30msec | 30msec wait for a response |
| 0x04 | 40msec | 40msec wait for a response |

In fact the Delay can be a product of the SubOpCodex10

i.e. Read time =  (SubOpCode X 10msec)

Examples

In red is the actual communication from the FMS sent to the Pump and this what the FMS will wrap in a protocol. What you see is 3 bytes being sent and there is a 4 byte response. This is one cycle of communication.

2018/12/05|15:04:05:873|COMM : COM7 -> (W) [3]  F1 40 B0

2018/12/05|15:04:05:912|COMM : COM7 -> (R) [4]  F1 40 B0 E1

When this data is wrapped in the protocol and sent by the FMS it will appear as;

32 01 03 12 02 03 03 F1 40 B0 33

Details explanation

| Byte# | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8-10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.g. | 32 | 01 | 03 | 12 | 02 | 03 | 03 | F140B0 | 33 |
| Desc | STX | ID# | Packet Control | Special Function | Sub OpCode | OpCode | P/Load Length | Payload | ETX |

The Packet Control: Directs the comms to Uart2. NB: This could be directed to Uart3, if the value was 0x04

Special Function:  If you look at this table you see that a byte value of 0x12 will causes the LEDS on both Tag Readers to Flash and activates Beeper 2 (only Short beeps). Now instead of using the regular Tagging protocol to communicate to the Tagging Section. This byte allows a communication to the Pump to also activate the specific Tagging functions. Should there be a Tag placed on the reader, the only indication of that will be in the StatusByte which could indicate that RFID1 or RFID2 has been presented with a Tag.  The FMS will then use the Tagging Protocol to retrieve the Tag.

SubOpCode: Determines the delay that uP will wait for the response. With each Byte, it will reset its timer to zero and wait for the next byte until the timeout period has elapsed .

OpCode: This instructs the uP to send the payload to the serial port and uses the SubOpcode as the timing delay to receive the response as mentioned above.

The rest is quite self explanatory.

The Response from the Pump

The response from the pump is then wrapped in the protocol and sent back to the FMS and it would looks like this;

32 01 03 12 02 03 04 F1 40 B0 E1 33

| Byte# | 1 | 2 | 3 | 4 | 5 | 6 | 7-10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.g. | 32 | 01 | 03 | 01 | 02 | 04 | F140B0E1 | 33 |
| Desc | STX | ID# | Sub OpCode | Status Byte | Payload Desc (Packet Control) | P/Load Length | Payload | ETX |

SubOpCode: This is probably not necessary, but it we will keep it in. But should we need another byte to send data back to the FMS, we could utilize this in the future.

StatusByte: Since the Special Function byte was used, this becomes important to know if the was Tag presented and 0x01 indicates that Tag1 has data.

NB: There is a Flag for RFID1 and RFID2 in the StatusByte, but in the Tagging Protocol, 0x01 is the only Tagging Flag as there protocol addressed each side individually and if the Flag is Set it is for the side being addressed.

References

| Byte | Description (from FMS) | Range |
| --- | --- | --- |
| 1 | STX: Start of Transmission | 0x32 |
| 2 | Tag ID # = R ID Number | 0x00 – 0x15 |
| 3 | Packet Control (directs complete to Uart2 or 3) etc | 0x00 - 0x01 Ignore 0x02 – Internal (like Tagging/ Mech Pump) 0x03 – Uart2 0x04 – Uart3 |
| 4 | Special Functions | See below |
| 5 | Sub OpCode | 0x00 – 0xFF |
| 6 | OpCode | 0x00 - Info 0x01 - Flash Memory 0x02 - FRAM Memory 0x03 - DigitalPump to Uart2 0x04 - Tagging 0x05 - Mechanical Pump 0x06 - Keypad/LCD 0x07 – Aux Comms Uart3 |
| 7 | Length of packet | 0x00 – 0xFF |
| 8 | ETX: End of Transmission | 0x33 |

| Packet Control Byte3 (directs complete to Uart2 or 3) etc | 0x00 - 0x01 Ignore 0x02 – Internal (like Tagging/ Mech Pump) 0x03 – Uart2 0x04 – Uart3 |
| --- | --- |

| Value | Desc Special Functions Byte 4 |
| --- | --- |
| 0x00 | Do nothing |
| 0x01 | Flash LED1 |
| 0x02 | FlashLED1 and Beep Tag1 |
| 0x10 | Flash LED2 |
| 0x20 | FlashLED2 and Beep Tag2 |

| Byte | Description (to FMS) | Range |
| --- | --- | --- |
| 1 | STX: Start of Transmission | 0x32 |
| 2 | Echo of ID # | 0x00 – 0x15 |
| 3 | Sub OpCode (echo back) | 0x00 – 0x04 |
| 4 | Status Byte * | 0x00 – 0x04 |
| 5 | Payload Description/(Packet Control) (02-Tag / 03-Pum | 0x02 – 0x03 |
| 6 | Payload Length | 0x00 -0x0a (10 = Tag#) |
| 7 | ETX: End of Transmission | 0x33 |

Revision This is the simplified Wrapped Protocol  5 November 2019

This will be a mix of the Pass-through method and the Wrapped protocol communication to the Pump.

When the MID receives a communication that is in this format;

32 01 03 12 02 03 03 F1 40 B0 33

| Byte# | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8-10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.g. | 32 | 01 | 03 | 12 | 02 | 03 | 03 | F140B0 | 33 |
| Desc | STX | ID# | Packet Control | Special Function | Sub OpCode | OpCode | P/Load Length | Payload | ETX |

The MID will notice that the Opcode redirects the payload.

OPCODE #3 = DIGITAL Pumps. This means that all the data in the Payload must be sent to UART2 as its configured baudrate (i.e. that of the Pump).

(NB: OPCODE #7 does the same to Uart3 but at 9600 )

All other Special Functions will work as defined and returned message from the Pump (if there is one) will be received in the same way as it does in the Pass-Through method and returned unwrapped to the Uart1.

That’s it!

Summary

The MID allows any comms not in the Tagging Style protocol to go immediately to Uart2.

All the Tagging Comms work as define previously.

All comms received by Uart2 (from the Pump) will be sent to Uart1 in the same way as in the Pass-through method.

If there is Tag data available, then the Tag Protocol returns the number and the Payload is dumped.