MID 7.2 DipSwitches Explained

Pinout on Dip switch 2
   *  1 MSB UART2 Baudrate
   *  2 MID UART2 Baudrate
   *  3 LSB UART2 Baudrate
   *  4 Protocol Wrapped ON/OFF (Repeat to UART 2 the unwrapped data)
   *  5 LCD Keypad ON/OFF side 1
   *  6 LCD Keypad ON/OFF side 2
   *  7 Mech pump Enable
   *  8 RFID ON/iButton OFF

 Pinout on Dip switch 1
   *  1 MSB #ID 
   *  2 MID MSB #ID
   *  3 MID LSB #ID
   *  4 LSB #ID
   *  5 The baudrate of UART 1 is copied from UART 2
   *  6 Auto clear protocol enable
   *  7-8 00 = Serial, 01 = RS485 and 10 = RF

Baudrate

| Sw1-3 | Baud Rate | Pump Make |
| --- | --- | --- |
| 000 | 1200, 8, E, 1 |  |
| 001 | 2400, 8, N, 1 |  |
| 010 | 4800, 8, E, 1 | Prowalco |
| 011 | 4800, 8, N, 1 |  |
| 100 | 5787, 8, E, 1 | Gilbarco |
| 101 | 9600, 8, N, 1 | Tokheim |
| 110 | 4800, 8, O, 1 |  |
| 111 | 38400, 8, N, 1 |  |