Training Centre document 2						17 Jan 2020

How to put the MID Test Rig into RS485 Mode (using the RS485 DB)

The Test Rig is configure with 4 MIDs using with one Tokheim, two Prowalcos and a Mechanical Pump.

This is brief guide on the steps to be taken to convert to RS485. The Configuration of the other MID switches are important but not covered here.

- Dipswitch1 – Set the Baudrate of  Uart1 of the MID to 9600 (RS485 Mode)

Sw7 = OFF and Sw8 = ON  (Hit the Reset button for changes to take)

- Select the Port of the RS485 PC Adapter and set it in Pumps (not necessary to configure the comm. port for Tagging)

- PumpController.ini

A)

Set all Pumps to: PortSettings          = 9600,N,8,1

For all Pumps that are configured.

Caution: All Pumps configured must have these same settings

B)  This values must still be played with find a suitable setting for all Pumps

;Delay values

PumpsPollDelay = 60

TaggingPollDelay = 45

C) Configure the MIDDual1 Settings

MIDMODE must be set to 1

- Fit the RS485 DB to the MID and Start the PumpController