# Board and programmer connector:

List of changes for HW V1.0

- 36 turns coil (30mm diameter or more)

- Add pull up to reset line

- Remove R3 and R11

- R1 must to be 0R

- R7 must to be 15k

# Programming sequence

- Open Atmel Studio

- Click on Device Programming

- Replace ATTiny104 by ATTiny102

- Click on “Apply”

- Read signature

- Click on Memories

- Open the file located on RFID_Reader_v1.0\Release\RFID_Reader_v1.0.elf

- Click on Program

- Check the Verification after programming