MID 7.2 DipSwitches Explained Rev 1.03

Pinout on Dip switch 2
   *  1 MSB UART2 Baudrate (Digital Pumps)
   *  2 MID UART2 Baudrate (Digital Pumps)
   *  3 LSB  UART2 Baudrate (Digital Pumps)
   *  4 ON - Enable Super Poll Mode / OFF – Normal Polling

*  5 ON - Super Poll Setup Clear / OFF – Super Poll Memory Mode

*  6 ON - Debug Console Start Message / OFF – Operational Mode

*  7 ON – Mechanical Pump Mode

*  8 ON – RFID Mode /OFF - iButton Mode


Uart2 Baudrates (Dipswitch 2)

| Sw1-3 | Baud Rate | Pump Make |
| --- | --- | --- |
| 000 | 1200, 8, E, 1 |  |
| 001 | 2400, 8, N, 1 |  |
| 010 | 4800, 8, E, 1 | Prowalco |
| 011 | 4800, 8, N, 1 |  |
| 100 | 5787, 8, E, 1 | Gilbarco |
| 101 | 9600, 8, N, 1 | Tokheim |
| 110 | 4800, 8, O, 1 |  |
| 111 | 38400, 8, N, 1 |  |