Master Controller – decoding the Transactional data in the MQTT Payload – 20 August 2019 
Now after a transaction the data returned is:

10:06:15.002 -> DB -> TRANSACTION: Number: 20

10:06:15.036 -> DB -> TRANSACTION: Datetime: 123456789

10:06:15.036 -> DB -> TRANSACTION: Nozzle Number: 1

10:06:15.036 -> DB -> TRANSACTION: Value expended: 554.79

10:06:15.036 -> DB -> TRANSACTION: Liters expended: 37.49

10:06:15.036 -> DB -> TRANSACTION: Price: 14.80

10:06:15.036 -> DB -> TRANSACTION: New Totalizer: 237.27

10:06:15.036 -> DB -> TRANSACTION: Vehicle TAG: 00 00 00 00 00 00

10:06:15.036 -> DB -> TRANSACTION: Driver TAG: 00 00 00 00 00 00

10:06:15.036 -> DB -> TRANSACTION: Attendant TAG: E4 DA BD 00 00 00

10:06:15.071 -> DB -> TRANSACTION: Odometer: 0

10:06:15.071 -> DB -> TRANSACTION: Hourmeter: 0

10:06:19.022 -> MQTT -> (W) MemIndex: 0 Topic: server Payload: 32 01 02 03 01 FA 0A 38

14 00 00 00 15 CD 5B 07 AF B2 0A 44 9A F1 15 42 CD CC 6C 41 E6 45 6D 43 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 E4 DA BD 00 00 00 01 01 00 00 00 00 33

10:06:19.229 -> MQTT -> Published

The transaction payload is 

14 00 00 00 15 CD 5B 07 AF B2 0A 44 9A F1 15 42 CD CC 6C 41 E6 45 6D 43 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 E4 DA BD 00 00 00 01 01 01 00 00 00

| Field Name | #ofBytes | Raw Data | Actual Data |
| --- | --- | --- | --- |
| Transaction# | uint32_t Transaction 4 | 14 00 00 00 | 20 |
| DateTime | uint32_t DateTime 4 | 15 CD 5B 07 | 123456789 (Here Hardcoded) |
| Value | float Value 4 | AF B2 0A 44 | 544.79 |
| Litre | float Liters 4 | 9A F1 15 42 | 37.49 |
| Price | float Price 4 | CD CC 6C 41 | 14.8 |
| Totalizer | float Totalizer 4 | E6 45 6D 43 | 237.27 |
| Odometer | uint32_t Odometer 4 | 00 00 00 00 | 0 |
| HourMeter | uint32_t Hourmeter 4 | 00 00 00 00 | 0 |
| Vehicle Tag | byte VehicleTAG 6 | 00 00 00 00 00 00 | 0 |
| Driver Tag | byte DriverTAG 6 | 00 00 00 00 00 00 | 0 |
| Att Tag | byte AttendantTAG 6 | E4 DA BD 00 00 00 | 000000BDDAE4 |
| Nozzle# | byte Nozzle_Number 1 | 01 | 1 |
| Tran Type | byte Transaction Type 1 | 01 | NORMAL |
| Trans_Status | byte Status 3 | 01 00 00 | NORMAL (Hardcoded) |
| Fuelling_Time | byte Fuelling_Time 1 | 00 | Not Active as yet |

Total bytes 56

Change Request

Change TranStatus to 1 Bytes and add 1 byte for the Fuelling Time Period.

This gives the number of minutes it took to do the transaction. The remaining 2 Bytes are Reserved.

Here is how it will apply;

17:08:21.014 -> DB -> ACTION: Storing Sale data
17:08:21.014 -> DB -> TRANSACTION: Number: 45
17:08:21.014 -> DB -> TRANSACTION: Datetime: 1167619115
17:08:21.061 -> DB -> TRANSACTION: Nozzle Number: 1
17:08:21.061 -> DB -> TRANSACTION: Value expended: 260.06
17:08:21.061 -> DB -> TRANSACTION: Liters expended: 17.80
17:08:21.061 -> DB -> TRANSACTION: Price: 14.61
17:08:21.061 -> DB -> TRANSACTION: New Totalizer: 511541.19
17:08:21.061 -> DB -> TRANSACTION: Vehicle TAG: 65 31 96 00 00 00 
17:08:21.061 -> DB -> TRANSACTION: Driver TAG: 00 00 00 00 00 00 
17:08:21.061 -> DB -> TRANSACTION: Attendant TAG: 00 00 00 00 00 00 
17:08:21.061 -> DB -> TRANSACTION: Odometer: 66887
17:08:21.061 -> DB -> TRANSACTION: Hourmeter: 0
17:08:21.061 -> DB -> TRANSACTION: Type: 1
17:08:21.061 -> DB -> TRANSACTION: Status: 1
17:08:21.061 -> DB -> TRANSACTION: Fuelling Time(min): 2


17:08:21.249 -> MQTT -> (W) MemIndex: 3 Topic: server Payload:

2D 00 00 00 2B 74 98 45 6C 07 82 43 66 66 8E 41 8F C2 69 41 A6 C6 F9 48 47 05 01 00 00 00 00 00 65 31 96 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 01 01 01 02 00 00

| Field Name | #ofBytes | Raw Data | Actual Data |
| --- | --- | --- | --- |
| Transaction# | uint32_t Transaction 4 | 2D 00 00 00 |  |
| DateTime | uint32_t DateTime 4 | 2B 74 98 45 |  |
| Value | float Value 4 | 6C 07 82 43 |  |
| Litre | float Liters 4 | 66 66 8E 41 |  |
| Price | float Price 4 | 8F C2 69 41 |  |
| Totalizer | float Totalizer 4 | A6 C6 F9 48 |  |
| Odometer | uint32_t Odometer 4 | 47 05 01 00 |  |
| HourMeter | uint32_t Hourmeter 4 | 00 00 00 00 |  |
| Vehicle Tag | byte VehicleTAG 6 | 65 31 96 00 00 00 |  |
| Driver Tag | byte DriverTAG 6 | 00 00 00 00 00 00 |  |
| Att Tag | byte AttendantTAG 6 | 00 00 00 00 00 00 |  |
| Nozzle# | byte Nozzle_Number 1 | 01 |  |
| Tran Type | byte Transaction Type 1 | 01 |  |
| Trans_Status | byte Status 1 | 01 |  |
| Fuelling_Time | byte Fuelling_Time 1 | 02 |  |
| Reserved | Byte Reservedd[2] | 00 00 |  |